Hello friends, welcome back to Malviya CAD Solution.

Today, we are going to design a cultivator, which is attached behind a tractor. We will create all of its parts one by one. I hope you will like this video. If you are new to our channel, please subscribe, and if you like the video, please like and share it.

So, let's start by preparing the frame, which we are going to create using the Weldment feature.

Frame

First, we will create the first part, which is the frame. We will change the units to IPS (inch-pound-second) and prepare the design in inches.

We will create a center rectangle here and set its dimensions to 82 inches and a height of 22 inches.

The central rectangle has now been created. We simply need to extrude it.

Now, the Weldment feature is visible here. If it is not visible, go to the Right-click menu and activate it.

Select Weldments, then go to Structural Member. Under Structural Member, select ANSI Inch.

For the frame, we need to use a tube, so we will select a square tube. Its size will be 0.1875 inches.

Next, enter the required group box, click here, and select the first line, followed by the second line.

We will now set the material to Satin Finish Stainless Steel and apply the appropriate color.

Next, select this face and create a sketch.

Show the previous sketch. After selecting it, choose Sketch → Convert Entities.

We are creating this geometry for construction purposes, so close the sketch.

According to the construction geometry, we now need to set the required dimensions and the hole size. The hole size is 12.5.

There is one hole here and another hole here. Simply press Escape to close the command.

Now we need to provide the dimensions. The hole size is 3 inches.

Select the distance feature. The feature is already selected. We now need to specify the direction, so select this line.

The distance is 9.5 inches.

We need 9 patterns. Select the pattern quantity and click OK.

In this way, the holes have been created on the frame.

Now select the sketch and go to Mirror. In Mirror, select the Front Plane and the required feature.

We need to create two brackets on this side where the attachment/fitting will take place.

For this, we will create a Corner Rectangle.

Its size will be 0.63 inches. The distance between both rectangles will be 1.5 inches, and the distance from the origin point will be 15 inches.

Now go to Features. Set the required depth, approximately 0.3 inches, and then change it to 0.4 inches. Click OK.

We need to apply a complete circular fillet.

Select the first face, second face, and third face. Similarly, apply the same operation to the other side by selecting the first, second, and third faces.

The first bracket on this side is now complete.

Now, using the Mirror Tool, we need to create the same geometry on the other side. Select the Right Plane.

Next, we need to create a hole.

Select Hole Wizard and choose the appropriate drill size. This time, the drill size is 25.5.

For the hole position, move the cursor over this curve. The automatic point indicator will appear. Simply click there and select OK.

The bracket is now complete.

We will save this part.

Let's create a new folder so that all our parts remain organized in one place.

Go to the Cultivator folder, create Part 1, and name it Frame.

Now close this part and move on to the second part.

Part 2 — Tine 01

Now we will create the second part of the cultivator.

First, change the units. Select the Right Plane.

We will position the geometry at the center of the origin point. Create a line.

Create another line and then enter the required dimensions.

The dimensions are:

- Length: 25 inches

- Dimension: 1.5 inches

- Distance between the two points: 2.25 inches

- Vertical distance: 2 inches

Apply all the required dimensions.

The height between these two points is 15 inches.

Apply the remaining dimensions. This dimension is 1 inch.

The sketch is now fully defined.

Apply 1.5 inches on the inside and 2 inches on the outside.

Create a rectangle with a width of 5 inches and a height of 1.25 inches.

The distance is 11 inches.

The height from this point to this point should be 22 inches.

Now create circles.

The hole size is 0.5 inches.

The distance between the two holes will be 3 inches.

Select the centerline and mirror the first and second holes about this line.

Use a blind condition with a depth of 0.4 inches.

The second part is now ready.

We can also apply a slightly darker appearance to it.

Save it as Part 2 — Tine 01.

Now close it and move on to the next part.

Part 3 — Tine 02

Now we are going to create the third part of the cultivator.

Again, change the units and apply all dimensions in inches.

First, select the Right Plane and create a line.

We will use a midpoint line.

Create another line and position it appropriately.

Now create the sketch using the required points.

The length of this line is 24 inches.

Close the lines on both sides.

The height from this point to this point is 2 inches, and the height on the other side is also 2 inches.

The gap between the two sides is 2.5 inches.

We will use a radius of 16 inches on both sides.

Another dimension is required from this point to the midpoint of the outer arc: 20.3 inches.

The height from this point to this point is 15 inches.

The sketch is now almost complete.

Apply the remaining dimensions:

- 1.5 inches

- 1.5 inches

- Distance between the points: 5 inches

- Top section: 1 inch

The sketch is now fully defined.

We need to create a flat section here.

Apply a 1-inch radius to this corner and click OK.

Go to Features.

The thickness will be 1.25 inches, the same as the previous part.

Click OK.

The bottom radius will be 2 inches.

Select the bottom face and create a sketch.

Create a center rectangle.

Create a centerline and make it vertical.

Select the appropriate point and line and apply the symmetry relation so that the rectangle is centered.

Now apply the dimensions:

- Width: 5 inches

- Height: 1.25 inches

- Distance from origin: 11 inches

- Center-to-center distance: 22 inches

Now create the center circles and holes.

Create one hole here and another here.

Hole diameter: 0.5 inches.

Select both holes using Ctrl and make them equal.

The distance between the two holes is 3 inches.

Select both holes and use Mirror Entities to copy them about the centerline.

Now apply a Blind condition with a depth of 0.5 inches.

The second tine is now complete.

Apply the appearance.

Select Metal → Steel → Sand Blast Steel, which is the same material we used previously.

Save it in the Cultivator folder as Part 3 — Tine 02.

Now close the part.

Clamp 01

We are now going to create the fourth part of the cultivator, which is the attachment clamp.

First, change the units.

Select the Right Plane and activate the Line Tool.

Create a line.

Make these two points vertically coincident.

Now apply the dimensions:

- Length: 4 inches

- Straight line length: 18 inches

- Dimension: 7 inches

- Angle: 45°

Delete the unnecessary line and make the required geometry vertically coincident with the origin.

Now apply a fillet.

The radius is 2 inches.

Select the required corners and click OK.

Next, use Offset Entities.

Set the offset to 0.5 inches, i.e. half an inch.

Select this line and set the direction toward the inside.

Close the sketch from both ends.

Now apply the required thickness/width of 3 inches.

For the rounded geometry, use the appropriate feature option.

Apply the same process on the other side.

Select the required faces and apply the necessary operations.

Now create the required hole with a 1-inch through condition and position it appropriately.

Click OK.

The first attachment clamp is now complete.

Apply an appearance of Satin Finish Stainless Steel and choose the desired color.

Save it as Part 4 — Clamp 01.

Now close it and move on to the second clamp.

Clamp 02

We will now create the second attachment clamp.

Change the units and again select the Right Plane.

Create a sketch and activate the Line Tool.

Create the required line.

The length of the line is 26.6 inches.

Create a centerline.

The tip angle will be 30°.

Apply the required fillet.

Then use Offset Entities, using the same 0.5-inch offset as before.

Close the sketch on both sides.

The sketch is now fully defined.

Just as in the previous part, apply the required rounded geometry on both sides.

Select the required faces and position them appropriately.

This part is now complete as well.

Apply the required appearance.

Blade 01

Now we will create the blade.

The dimensions are 13 × 4 inches, with a thickness/height of 0.5 inches.

Select the top face and create a sketch.

Create a centerline.

Create another centerline.

Now create the required line.

Create the sketch accordingly.

Apply the required dimensions:

- Angle: 13°

- Gap: 1.5 inches

Now apply the required flat/feature operation.

Select the required geometry about this line.

Go to Mirror.

Select the required geometry and mirror it about the centerline.

Now go to Features and click OK.

The blade is now ready.

Activate the Line Tool and connect these two points.

Click OK.

Now select this line.

Using Ctrl, select the three required lines and apply Convert Entities.

The sketch will be created.

Go to Features and use the required Extruded Cut operation.

The cut will be created as shown.

Now apply a complete round fillet.

Select the first, second, and third faces.

Repeat the same operation on the other side.

Click OK.

Now apply a Normal Plate operation with a thickness of 0.2 inches.

Select all the required faces, including the back side, and click OK.

The blade is now complete.

We now need to bend it.

At the moment, the blade is parallel, so we need to bend it slightly.

For bending, go to the menu bar, then Insert → Features → Flex.

Open the Flex feature.

Select the blade as the component and choose Bending.

First, select the appropriate plane/point.

Select the first vertex and then the second vertex.

We can bend the blade from here as well.

Set the bending dimension to -10 inches.

Click OK.

As you can see, the blade has now been bent properly and is ready to be installed on the cultivator.

Save it as Part 6 — Blade 01.

Blade 02

Now we need to create another blade because our two tines have different radii.

One has a 10-inch radius, while the other has a 16-inch radius.

We can simply edit the existing Flex feature.

Edit the Flex feature and change -10 to -16.

As you can see, the radius has now changed to 16 inches.

Save the part.

Now we need to create the assembly.

Assembly

Go to New and select Assembly.

Activate the origin point and mate the required components.

Then go to the appropriate view and close the origin display.

Now select the background/reference component.

Go to Insert Components.

Open the first tine.

Apply the required mates.

Select the bottom face and the corresponding face, then click OK.

Select the required holes and mate them properly.

Select the appropriate relation and make the holes concentric.

The first tine is now properly positioned.

Next, select Linear Component Pattern.

For the direction, select this line.

Set the distance to 19 inches and the number of patterns to 5.

Reverse the direction and click OK.

Now go to Insert Components again.

Open the second tine.

Apply the required mates and position its holes correctly.

The second tine is now properly positioned.

Again, activate Linear Component Pattern.

Select the required direction and reverse it if necessary.

Select the required component.

All the tines are now positioned.

We will also set the color/appearance of the tines so that they have the same appearance and the overall design looks better.

Applying the correct appearance is important because it makes the project look more professional and visually appealing.

Now the tines are properly created and assembled.

Again, go to Insert Components and open the first blade.

Select the required mates and position the blade properly.

Next, open the tine part.

Select its Right Plane.

Open the blade and select its Front Plane.

Apply the appropriate mate.

Select the midpoint of the tine.

Set the distance to 3 inches and click OK.

The blade is now properly positioned.

Go to Linear Pattern.

Select the required line and reverse the direction as necessary.

Set the required number of patterns.

Now repeat the same process for the second blade.

Open the appropriate part, select its Right Plane, and select the blade's Front Plane.

Apply the required mate and set the distance to 3 inches.

The second blade is now properly positioned.

Again, use Linear Component Pattern as required.

Now insert the attachment clamp.

Open the attachment clamp and apply the required mates.

Open the frame part.

Select the appropriate plane, such as the Front Plane.

Open the clamp that we just inserted and select its appropriate plane, such as the Right Plane.

Apply the mate.

The clamp is now properly positioned.

Using Ctrl, select and copy the required component.

Now select the appropriate holes and mate them so that they are properly centered.

As you can see, the cultivator assembly is taking shape.

Select the required faces and drag/copy the component to create the corresponding components on the other side.

Now select the Cultivator Assembly.

The frame can also be edited directly within the assembly.

Select the required face and create a sketch.

Select the required geometry and activate Trim Entities.

The sketch is now complete.

This is the central hole. Apply Convert Entities to it as well.

Apply the required offset and reverse the direction where necessary.

Use Mirror about the Right Plane.

Apply the required geometry pattern.

Now apply the required flat/feature operation.

As you can see, the cultivator assembly is now complete.

We now need to insert the bolts.

Use the required hex bolts.

Set the required size.

Go to the Toolbox and insert the required hardware component.

Set the length to 1.5 inches.

The first bolt is too long, so we will edit it and make it shorter.

Set the required length to approximately 2.75 inches.

The other bolt needs to be made longer.

Edit it through the Toolbox and set its length to 5.5 inches.

First, reduce the length of the bolt and fit it into position.

Set the required dimension to 1.25 inches.

The required length is 5 inches.

Another component will be set to 4 inches.

Edit it as required.

Go back to the Toolbox.

Open the required component and set its size.

We need 1.25 TPI, with the required double-chamber/symmetric configuration.

Click OK.

Place the component in the required position. The center will be automatically located.

Again, go to Toolbox Components.

The required hardware will now be positioned.

Select the direction by selecting this line.

The gap is 9.5 inches.

Set the number of component patterns to 9.

The bolts are now properly positioned.

Again, go to Linear Component Pattern and then select Mirror Components.

Select the appropriate plane, such as the Front Plane.

Select all the bolts carefully.

Make sure that every required bolt is selected.

Click OK.

As you can see, all the bolts have now been added.

The cultivator is now completely ready.

I hope you liked this project.

If you enjoyed the project, please like the video, comment, and share it.

We regularly create and share similar projects and video tutorials for you.

Stay connected with our channel, and we will meet again soon in the next video.

Until then, goodbye!